EXPERIMENTAL

80170NX
ELECTRICALLY TRAINABLE
ANALOG NEURAL NETWORK

¥	Ultrahigh Performance

Ñ	2 Billion Connections Per Second (CPS)

Ñ	Low-Power CHMOS*III EEPROM Technology

Ñ	300K Patterns Per Second (64- or 128-input Patterns)

¥	Versatile Architecture

Ñ	64 Dot-Product Neurons with Sigmoid Functions

Ñ	Separate Input and Feedback Arrays

Ñ	Hold and Feedback Capability

Ñ	Supports Hopfield, Multi-Layer, and Recurrent Nets

Ñ	10,240 Nonvolatile Analog Synapse Weights

Ñ	6- to 7-bit Typical Resolution

¥	Compatible with Popular Training Methods

Ñ	Back-Prop and Competitive Learning

Ñ	Single-Neuron Perturb for Madaline III Learning

¥	Flexible I/O

Ñ	64 Physical Inputs and Outputs, Configurable for 128 Inputs

Ñ	Supports I/O Cascading

Ñ	I/O Referencing Supports TTL Operation

Ñ	User-Controlled Gain on Neuron Sigmoid Functions

¥	Many Real-Time Applications

Ñ	Signal Processing

Ñ	Process Optimization

Ñ	Robotic Motion

Ñ	Associative Memory

¥	208-Pin PGA Package

(See Packaging Specification, Order #231369)

Table of Contents

General Description	1

PIN DESCRIPTIONS	3

PRINCIPLES OF OPERATION	5

PROCESSING CONFIGURATIONS	7

MULTI-CHIP CONFIGURATIONS	13

SYNAPSE CHARACTERISTICS	15

Synapse Weight Operation	15

Synapse Multiply Characteristics	16

MODE DESCRIPTIONS	16

Signature Read	16

PDPÑParallel Distributed Processing Mode	16

WMI	16

WVTR	17

RMI	18

SMR	19

SYNR	21

NR	21

Mode Summary	23

TRAINING OPERATIONS	26

Training Systems	26

Multi-Chip Training	26

Chip-In-Loop Training	26

Weight Setting	26

Madaline Learning	30

Function Simulation	30

Bake-Train-Bake	31

Temperature and VCC Performance	32

performance	33

Weight-Cycling Performance	33

Performance After Radiation Exposure	34

ABSOLUTE MAXIMUM RATINGS	36

OPERATING CONDITIONS	37

DC CHARACTERISTICS	37

AC CHARACTERISTICS	38

CAPACITANCE	38

Pin Assignments	41

ORDERING INFORMATION	41

REFERENCES	42

General	42

Cognitive Psychology	42

Neurobiology	43

Neural Network Models and Learning	43

Speech Recognition	43

Temporal Pattern Recognition	43

Implementation	44

Control	44

Glossary	45

Index	47

Table of Figures

Figure 1. 80170 NX Chip Block Diagram	2

Figure 2. Model of Biological Neuron Cell	5

Figure 3. Typical Neural Net	6

Figure 4. Synapse Implementation	7

Figure 5. Input Multiplexing Circuits	8

Figure 6. 64-Input Feedforward Configurations	9

Figure 7. 128-Input Feedforward Configurations	10

Figure 8. 64-Input Feedback Configurations	11

Figure 9. 64-Input Sequence Recognition and Generation Configurations	12

Figure 10. 128-Input Feedback Configuration	13

Figure 11. Multi-Chip Configurations	14

Figure 12. EEPROM Floating-Gate Cell Characteristics	15

Figure 13. Four-Quadrant Synapse Multiply Characteristics vs. InputÊVoltage	16

Figure 14. VT Measurement System	17

Figure 15. Synapse Four-Quadrant Multiply Characteristics vs. VT Differential	18

Figure 16. Non-Linear Resistive Summing Load	19

Figure 17. Summing Nodes Selection and Output	20

Figure 18. Summing Node Output Measurement	20

Figure 19. Neuron Output Transfer Characteristic Vs. SummingÊCurrentÊandÊVGAIN	21

Figure 20. Neuron Output Transfer Characteristic Vs. SummingÊCurrentÊandÊVREF	22

Figure 21. Sigmoid Characteristic in VGAIN and HGAIN Modes	22

Figure 22. VPP2 Voltage Curves for Positive ÆVT Ñ Program	27

Figure 23. VPP2 Voltage Curves for Negative ÆVT Ñ Erase	28

Figure 24. Weight-Setting Algorithm	30

Figure 25. Reduction of VT Shifts Through Bake-Train-Bake	32

Figure 26. Sigmoid Temperature Sensitivity	32

Figure 27. Sigmoid VCC Sensitivity	33

Figure 28. Misclassified Patterns and MSE After Radiation Exposure	35

Figure 29. Weight Distributions After Radiation Exposure	35

Figure 30. AC Waveforms for Parallel Processing	39

Figure 31. AC Waveforms for QUERY Modes	39

Figure 32. AC Waveforms for Weight Modify	40

Figure 33. 208-Pin PGA Package	40

# General Description

The 80170NX Electrically Trainable Analog Neural Network (ETANN) chip offers unprecedented single-chip bandwidth and storage capacity for pattern-recognition problems. The chip simulates the data processing functions of 64 neurons, each of which is influenced by up to 128 positively or negatively weighted ÒsynapseÓ inputs. The chipÕs data inputs and outputs are analog; its control functions for setting and reading synapse weights are primarily digital.

As shown in Figure 1, inputs and synapse weights are organized in two processing arraysÑa 64x64 Input Array and a 64x64 Feedback Array. The chip has 64 external analog inputs. These inputs can be routed either to the Input Array or to the Feedback Array. Using input-hold functions, sets of 64 inputs can be multiplexed to the two arrays for mapping 128 inputs into 64 neuron outputs. Using these inputs, clocked feedback functions can support many interesting processing options.

Both the Input and Feedback Arrays are accompanied by a 16x64 bias array, for a total of 10,240 synapses. The chip computes the dot product (inner product) of the analog inputs and the weights stored at the synapses. The results are summed along each of the 64 neurons, generating 64 scalars that are passed through a sigmoid function of each neuron to the outputs.

Fully-parallel processing using both the Input and Feedback Arrays yields performance in excess of 2 billion multiply-accumulate operations (connections) per second (CPS), well beyond the capability of conventional CPUs. By interconnecting eight 80170NX chips, systems can achieve more than 16 billion CPS, a performance level that exceeds most supercomputers.

Multiple neural network layers can be implemented with either a single 80170NX chip or with multiple chips. By segmenting the two 64x64 arrays, networks with more than two layers can be implemented on a single chip. Multiple 80170NX chips can be cascaded by feeding analog outputs of one chip directly into analog inputs of subsequent chips.

Interfacing flexibility is provided through voltage-controlled I/O referencing and sigmoid gain controls. A high sigmoid-gain control (HGAIN) supports fully digital 0-to-5-volt interfacing directly to digital CMOS or TTL logic.

The four-quadrant multiplier synapses, implemented with IntelÕs CHMOS*III EEPROM technology, provide truly nonvolatile, reprogrammable weights without refreshing or battery-backup. EEPROM reprogrammability supports iterative and application-specific learning, avoiding mask-process limitations.

A glossary of neural networks terms is included at the back of this data sheet for your convenience.

Figure 1. 80170 NX Chip .i.Block Diagram

# PIN DESCRIPTIONS

| Symbol | Type | Description and Function |
| --- | --- | --- |
| I0-I63 | IA | NEURON INPUTSÑfully buffered analog channels for stimulus-vector input, where input voltages range from 0V to VIA (see D.C. Characteristics). Input voltages above VREFi represent positive numbers while those below VREFi represent negative numbers. Inputs and stored synapse weights can be either positive or negative. Four-quadrant multiplications are performed on the inputs and stored synapse weights. Neuron inputs drive the Input Array and, for 128-input operation, can also be multiplexed to drive the Feedback Array , in which case VREFo defines ± inputs. |
| N0-N63 | I/OA | NEURON OUTPUTS and FEEDBACK INPUTSÑfully buffered analog outputs, representing neuron activity levels, ranging from 0V to 2*VREFo (see D.C. Characteristics). For 128-input operation, these same pins can be driven externally for clocked Feedback-Array input when outputs are disabled (see NE#). |
| CE# | ID | CHIP ENABLE #Ñan active-low TTL input that enables the chip when CE#Ê=Ê0. When CE#Ê=Ê1, the chip is put into a power-down or standby state, and outputs N0-N63 are disabled. |
| NE# | ID | NEURON ENABLE #Ñan active-low TTL input that enables neuron outputs N0-N63 when NE#Ê=Ê0. Output drivers are disabled when NE#Ê=Ê1, allowing several 80170NX chips to share a bus. NE# allows clocking of data into the Feedback-ArrayÕs sample and hold buffers from an off-chip source. |
| VREFi , VREFo | IA | INPUT AND OUTPUT REFERENCE VOLTAGES Ñvoltage-reference inputs that define the 0 or null voltage levels independently for inputs and outputs. Voltages above these references represent positive numbers while those below are negative. The maximum output voltage is 2*VREFo (see D.C. Characteristics). Setting VREFi and VREFo to 1.4V allows TTL compatible operation. VREFo is equivalent to the Feedback ArrayÕs VREFi. |
| VGAIN | IA | GAIN CONTROL VOLTAGEÑa voltage-reference input for controlling analog sigmoid gain. An input voltage from 0V to VCC adjusts the sigmoid gain from the minimum to the maximum. sigmoid response to VGAIN saturates at about 3.5V. All neuronsÕ gains are set simultaneously. |
| HGAIN | ID | HIGH GAIN MODEÑa TTL input that turns on the high-gain sigmoid characteristic when brought active-high. This in effect converts the sigmoid into a comparator for 0V or 5V output. |
| HOLD | ID | HOLD INPUTÑa TTL input that causes neuron inputs to be sampled and held when taken high. Input levels are captured and capacitively stored on the rising edge of HOLD. The Input-Array inputs are driven by this stored value when HOLD is high. I0-I63 resume driving the Input Array directly when HOLD goes low. |
| RESETF | ID | RESET FEEDBACKÑa TTL input that resets the Feedback-Array inputs and associated bias inputs to 0V when brought active-high. This effectively disconnects the Feedback Array from the neuron summing columns. |
| RESETI | ID | RESET INPUTÑa TTL input that resets the Input-Array inputs and associated bias inputs to 0V when brought active-high. This effectively disconnects the Input Array from the neuron summing columns. |
| CLK | ID | CLOCKÑthe TTL input that causes outputs to be sampled and held on the rising edge of CLK. Outputs are never connected directly back to inputs; they are only transferred synchronously in successive CLK cycles. When A12=1, the sample function in the input buffers for the Feedback Array is disabled. The CLK pin then behaves like the HOLD pin, but with inverted polarity. In addition, the feedback path is disconnected, and the input pads are multiplexed into the Feedback-Array input buffers. |
| A12 | ID | INPUT MULTIPLEXERÑin the forward-propagation mode, address pin A12 has the function of enabling the multiplexing of input pins into the Feedback Array. When A12 is brought active-high, the path from output pads to the Feedback Array is disconnected, while a path from the input pins to the Feedback Array is enabled. At the same time, the function of the CLK pin is modified so that it behaves like an inverted-polarity HOLD pin. |
| A12-A0 | ID | ADDRESSESÑTTL-compatible inputs used to select the individual synapses and neurons. The lower 6 bits (A5-A0) are column/neuron addresses and the upper 7 (A12-A6) are row/input addresses, which together select one synapse. A12Ê=Ê1 also selects the Feedback Array for input through the neuron inputs, I0-I63. |
| LRN | ID | LEARN CONTROLÑa TTL input for enabling the learning modes used to change the values of the synapse weights. |
| SSYN | ID | SYNAPSE SELECTÑa TTL input that enables the row-decoding circuitry for applying program, erase, or input voltages to the row selected by A12-A6 and BIAS pins. SSYN also activates the column-select circuitry for accessing a single synapse (per addresses A5-A0) through the neuron and summing-node multiplexers in SYNR (Synapse Read) mode. |
| WT | ID | WEIGHT-CELL SELECTÑa TTL input for selecting one or the other of the two EEPROM cells in the selected synapse (referred to as the weight EEPROM cell or the reference EEPROM cell). WTÊ=Ê1 selects the weight EEPROM cell, WTÊ=Ê0 selects the reference EEPROM cell. |
| INC | ID | INCREASE or DECREASE SYNAPSE WEIGHTÑa TTL signal that specifies whether to increase (INCÊ=Ê1) or decrease (INCÊ=Ê0) the threshold voltage of the floating-gate EEPROM cell selected by the WT pin. Increasing a synapse weight corresponds to decreasing (erasing) the threshold of the corresponding weight EEPROM cell and/or increasing (programming) the threshold of the corresponding reference EEPROM cell. Similarly, decreasing a synapse weight corresponds to increasing the threshold of the weight EEPROM cell and/or decreasing the threshold of the reference EEPROM cell. The difference in threshold voltages between the weight and reference EEPROM cells is called VT_DIFF. The relationships are: Weight =Ê[VT_REF-VT_WT] and VT_DIFF = - Weight Typically, the weight-setting algorithm maintains a constant common-mode threshold voltage for all EEPROM cells. |
| BIAS | ID | BIAS-SYNAPSE SELECTÑa TTL input to select the nonvolatile bias synapses. It is used in conjunction with the A5-A0 addresses to select one of 64 neurons and with A9-A6 addresses to select one of 16 rows of bias synapses. A12 determines the group of 16 rows selected: A12Ê=Ê0 for input bias, A12Ê=Ê1 for feedback bias. Bias-row addresses 10-15 are reserved for Intel internal use. Users should make sure that these bias addresses are zeroed if using custom software. |
| VPP1, VPP2 | SA | PROGRAMMING VOLTAGE SUPPLIESÑVPP1 and VPP2 are high-voltage pulse inputs required for training (see A.C. and D.C. Characteristics and Weight Setting). VPP1 and VPP2 are set to VCC during normal operation. They are connected to the gate and source of the EEPROM device during the WVTR (Weight VT Read) or RVTR (Reference VT Read) modes. |
| QUER | ID | QUERYÑa TTL input that enables the query mode when QUERÊ=Ê1. QUER activates interrogation of synapse weight and sum measurement. |
| SMO+, SMOÐ | OA | SUMMING-NODE OUTPUTSÑdifferential analog outputs of the neuron summing nodes in the SMR (Sum Read) mode. SMO+ and SMOÐ are connected to the selected neuron by the A5-A0 addresses for direct measurement of the differential voltage representing: Netj=Ê(SMO+)ÊÐÊ(SMOÐ)Ê=Ê·{weights*inputs}Ê+Êbias. Differential currents can also be read, but only for a very limited range. In the SYNR (Synapse Read) mode, addresses A12-A0 and BIAS select an individual synapse to obtain its response to a corresponding input. The output is a differential voltage or current across the SMO± pins. SMO+ and SMOÐ are also used in the SMR/P (Sum Read/Perturb) mode to allow adding or subtracting incremental differential current from a selected neuronÕs summing nodes for use with the Madaline learning algorithm (see Training). |
| NMO | OA | NEURON OUTPUTÑa multiplexed analog output connected to the neuron output selected by addresses A5-A0 in the NR (Neuron Read) mode. Selective sensing of neuron outputs facilitates implementations of back propagation and other learning algorithms with minimal hardware overhead. NMO is essential for system debugging. |
| SYNO | OA | SYNAPSE OUTPUTÑan analog output connected to one of the selected synapseÕs two EEPROM-cell drains for threshold measurement in WVTR, RVTR, or VT Read modes. The synapse is selected with the A12-A0, BIAS and WT inputs (see Weight Setting). |

KEY:
SÊ=ÊPower Supply

IÊ=ÊInput

OÊ=ÊOutput

AÊ=ÊAnalog

DÊ=ÊDigital

# PRINCIPLES OF OPERATION

Figure 2 is a simplified model of a typical biological neuron cell. The neuron has several inputs attached to its body at points called Òsynapses.Ó The variable-strength synapse connections can be either output-exciting or output-inhibiting. Input signals reaching the neuron body through these variable-strength synapses are summed by the neuron body. When the sum exceeds a threshold, the neuron output is activated.

Figure 2. Model of Biological Neuron Cell

The 80170NX chip is designed around this basic neuron model. The chip normally operates as 64 parallel processors, where each such processor is analogous to this neuron model. Figure 3 illustrates the 80170NX chip architecture as an array of external inputs (ui) and neuron outputs (vj). The intersections of external inputs and neurons are connected through synapses that consist of a multiplier (X) and a weight-storage unit (Wij). Wij is the weight of the connection from input ui to output vj. Figure 3 illustrates only one array of intersections; the chip has two such arraysÑan Input Array and a Feedback Array. In addition to these arrays, each neuron is connected to fixed-input bias synapses (Bkj) in each of the two arrays.

This normal mode of operation is called Parallel Distributed Processing (PDP). In the PDP mode, each neuron computes the dot (inner) product of the external inputs (ui) times their corresponding synapse weights (Wij), and adds the sum of the fixed-input bias weights (Bkj). This dot product is then compressed through a sigmoid function, as follows:

vjÊ=Êsigmoid {·i (ui * Wij) + ·kBkj}

The neuron output (vj) is a scalar quantity that indicates how closely the input vector matches the stored-weight vector. The sigmoid function through which the result of the dot products pass is illustrated later in Figures 20, 27 and 28.

The Input and Feedback Arrays each have 16 fixed-input bias synapses. Mathematically, all of these bias synapses act as a single bias to provide the threshold for the neuronÕs sigmoid function and to cancel offsets due to physical non-linearities in the chip. However, since the dynamic range of a single EEPROM bias synapse is limited, 16 of them are used. All 16 contribute to the dot product performed by the neuron, although seven are set by the training program and the other nine are set by a chip-initialization process.

The weights of all synapsesÑexternal-input synapses and fixed-input bias synapsesÑare programmed by an external training system. Intel provides such a training system, as described in the TRAINING OPERATIONS section on page 26 <****replace page number****>.

Figure 3. Typical Neural Net

The chip uses IntelÕs fast CHMOS-III EEPROM technology. Synapses are implemented as differential EEPROM cell pairs, as shown in Figure 4. This implementation provides much more compact storage than conventional digital EEPROM memory because synapse weights are stored as analog values on the gates, and because analog multipliers are more compact than digital multipliers. Typical weight resolution of the differential EEPROM cell pairs is 6 to 7 bits. Worst-case resolution over a lifetime of 10 years is at least 4 bits, and greater than 6 bits can be obtained if the Bake-Train-Bake Method is used.  (See the Bake-Train-Bake section on page 31 <****replace page number****> for details.)

Synapse weights are stored as analog transconductance values, each producing an analog output current from an analog input voltage and a stored-weight voltage. Currents generated by each synapse along a pair of summing lines (shown as a single column in Figure 3) are summed, converted to a voltage, and passed through a sigmoid function with voltage-controlled gain. When both Input Array and Feedback Array are active, the output corresponds to the sum of their two dot products. Identical mechanisms in each of 64 columns correspond to the 64 neuron outputs. The sigmoid function (illustrated in Figure 19) is a threshold function where dot products below the neuronÕs output threshold indicate a poor match between input and stored weight vectors. Conversely, neuron output will be high for dot products above the neuronÕs threshold. Each neuronÕs threshold can be changed independently by the learning algorithm, allowing different match criteria for different vectors. Changing bias-synapse weights alters the sum of currents, shifting the neuron threshold.

Figure 4. .i.Synapse Implementation

As Figure 4 illustrates, each synapse contains a multiplier and a stored weight, and calculates one product. The synapse product is summed with those from other synapses along the summing lines I+ and I- to produce the dot (inner) product. The weights are stored as a difference in floating-gate thresholds of a pair of EEPROM cells. Changing weight values involves altering the charge stored on the floating gates. These modifications represent variable-weight strength as well as polarity. Since the floating gates of an EEPROM cell pair have equal capacitance, unequal charge causes differences in floating-gate voltages.

The synapseÕs multiplier circuit generates a differential current proportional to the product of this differential floating-gate voltage, ÆVFG, and the differential-input voltage, ÆVIN. The larger the differential voltages, the larger the differential current generated. Thus, the input and weight products are mapped into associated differential currents. Taking advantage of KirchoffÕs Law, many such currents can be summed along a pair of bit lines. The sum is converted to a voltage by load devices attached to the bit lines.

# PROCESSING CONFIGURATIONS

The chip supports many neural network configurations. For example, the 64 analog input levels are capacitively stored for a limited time by taking the HOLD pin high. Several 80170NX chips can share an input bus, capturing different time slices by sequential activation of the HOLD function. Many forms of signal processing (such as speech processing) can use this sampling method in which input data is gathered from different points in time.

Similarly, 64 analog voltages can be directed to the lower half of the synapsesÑthe Feedback Array. Figure 5 shows a simplified block diagram of the input multiplex circuits. The path from the input pins I0-I63 to the Input-Array buffers is always connected. These 64 signals can be multiplexed from the input pins to the Feedback-Array buffers. In the Parallel Distributed Processing (PDP) mode, the A12 address pin enables input multiplexing by creating a path from the input pins to the Feedback Array buffers, and disables the path from the output pins to the Feedback Array buffers. The HOLD and CLK signals control the propagation of the input signal to the Input Array and Feedback Array, respectively. Similarly the RESETI and RESETF enable their respective arrays.

Figure 5. .i.Input Multiplexing Circuits

Figures 7 through 11 illustrate some basic processing configurations. The simplest example is a 64-input single-layer feedforward network, shown in Figure 6(A). Here, 64-dimension input vectors (or ÒpatternsÓ) are directly mapped into 64-dimension output vectors, as determined by the synapse weights in the Input Array. In this minimal configuration, the Input-Array inputs are enabled and the Feedback-Array inputs are disabled by taking the RESETF pin high. This is called the 64-input parallel distributed processing mode (PDP/64-input mode).

Figure 6(b) shows another variant of the PDP/64-input feedforward configuration. Here, the Feedback Array is driven directly from the external input pins by taking A12 high, disabling the Input Array by taking RESETI high,  and enabling the Feedback Array by taking RESETF low.

Figure 6. .i.64-Input Feedforward Configurations

In addition to the chipÕs 64-input configuration (PDP/64-input mode), a 128-input configuration (PDP/128-input mode) can be implemented. In this configuration, shown in Figure 7, a second set of 64 inputs on input pins I0-I63 is multiplexed to the Feedback Array. Each of the 64 neurons then calculates the dot product of both the Input-Array and Feedback-Array input vectors with their corresponding synapse weights. Pin A12 enables this multiplex mode. When A12 and CLK are set TTL-high and RESETI, RESETF, and HOLD at TTL-low, the first half of the input vector is applied to the input pins and routed to both the Input-Array buffers and the Feedback-Array buffers. When HOLD goes to TTL-high, the Input-Array buffers store the analog levels and disconnect from the input pins. The second half of the input vector is applied to the input pins that propagate to the Feedback Array, and a valid output will appear at the output pins N0-N63 one processing-delay later.

Figure 7. .i.128-Input Feedforward Configurations

A single chip can perform two-layer operation by using the feedback connections and the clocked sample-and-hold features. Alternate activation of the two reset pins, RESETI and RESETF, disables the corresponding Input-Array and Feedback-Array inputs, as shown in Figure 8(A). During Input-Array processing, Feedback-Array inputs are reset by taking RESETF high. Outputs then reflect a dot product of inputs times the synapse weights stored in the Input Array. After a CLK pulse, which initiates processing by the Feedback Array, neuron outputs are fed into the feedback inputs of the Feedback Array. To prevent the external inputs at input pins from interfering with Feedback-Array processing, inputs are disconnected from the Input Array by taking RESETI high. The fixed internal inputs to the bias synapses are also reset when RESETI or RESETF are taken high.

Figure 8. .i.64-Input Feedback Configurations

Hopfield networks are supported with the clocked feedback capability shown in Figure 8(B). This requires applying alternating CLK and NE# pulses. Continuous feedback is also supported and can be obtained by driving the A11 pin to 12VÊ±Ê0.5V, the A12 pin to 0V and the CLK pin to TTL high.

Feedback connections can be used to recognize time-dependent input sequences, as in Figure 9(A). Here, results from the Input Array are fed back for multiple clock cycles to the Feedback Array. Figure 9(B) shows a configuration capable of generating sequences from a fixed input pattern.

Figure 9. .i.64-Input Sequence Recognition and Generation Configurations

Figure 10 combines the 128-input PDP mode with two-layer processing. The Input and Feedback Arrays are combined to allow up to 128 inputs and up to 64 outputs, as shown in Figure 10(B). If this 128-by-j matrix is used for the first layer of processing then a j-by-(64-minus-j) matrix located in the Feedback Array, as shown in Figure 10(B), can be used for the second layer of processing. The number of processing elements in the hidden layer, j, is defined as an integer less than 64. The outputs of the first cycle are clocked back into the Feedback Array; only the first j outputs of this cycle are valid. These outputs are multiplied by the j-by-(64-minus-j) matrix in the second cycle. Then, the outputs of the second cycle are read at the output pins NjÐN63. Those portions of the synapse array outside the two matrices defined above, marked by 0 in Figure 10(B), must be set to weights of zero.

Figure 10. .i.128-Input Feedback Configuration

# MULTI-CHIP CONFIGURATIONS

One of two multi-chip interconnection configurations can be used: direct pin-to-pin interconnection, as shown in Figure 11(A), or bus interconnection, as shown in Figure 11(B). Direct pin-to-pin wiring is the more straightforward since no multiplexing is required. The busing example in Figure 11(B) shows a common, multiplexed bus for all communication. Alternatively, separate buses could be used between processing layers and for inputs and outputs. Bus interconnection provides a more flexible development environment; the clocking sequence of the NE#, HOLD, and CLK pins allows changes in data routing. The bus configuration has two drawbacks compared to direct interconnect: (1) data can be moved only in complete 64-neuron blocks, and (2) multiplexing is somewhat slower.

Intel offers a multi-chip prototyping board for 80170NX chipsÑthe ETANN Multi-chip Board (EMB). The EMB supports both pin-to-pin and bus interconnection along with prototyping for up to eight chips. It is an add-on to the iNNTS training system, described below in the TRAINING section. Multi-chip versions of the required hardware drivers, linking subroutines, and network simulation programs are provided.

Figure 11. Multi-Chip Configurations

# SYNAPSE CHARACTERISTICS

## Synapse Weight Operation

To change a synapse weight, the difference in voltage between the two EEPROM floating-gate cells (ÆVFG) is changed by adding charge to one cell and removing charge from the other using Fowler-Nordheim tunneling. Tunneling is a physical mechanism commonly used in conventional EEPROM cells; high voltage applied across a thin dielectric causes electrons to tunnel directly through the dielectric, to or from a floating gate. The weight-modification voltage pulses induce Fowler-Nordheim tunneling in a selected synapse (see Figure 4).

A measurable indicator of the floating-gate voltage is the threshold voltage (VT) of the floating-gate cell itself. Figure 12 shows the floating-gate cellÕs current-voltage characteristics before and after a weight modification, along with the corresponding VT's. The VT is the control-gate voltage at which the cell begins conducting a given amount of current, typically 1 to 10 µA. In this example, the floating-gate voltage becomes more negative as electrons are added, corresponding to an increase in the cell VT, since a more positive control-gate voltage is required to overcome the effect of the added electrons on the floating gate.

Figure 12. .i.EEPROM Floating-Gate Cell Characteristics

Synapse weights are represented by threshold-voltage differentials (VT_DIFF) between the reference and weight EEPROM cells, where weight =Ê [VT_REF-VT_WT] and VT_DIFF = - weight. All weight-changing procedures refer to VT changes, which are measured by using the VT read modes, WVTR and RVTR, as described in the MODE DESCRIPTIONS section, below.

## Synapse Multiply Characteristics

Figure 13 shows the four-quadrant multiply characteristics of a synapse. The output differential current is plotted as a function of the input voltage for eleven values of VT_DIFF. A constant common-mode VT among the two EEPROM cells is maintained in these results. There is good linearity over most of the weight range. (See the DC CHARACTERISTICS section on page 37 <****replace page number****> for a definition of VT_DIFF, VT_COM, VT_WT, and VT_REF.)

Figure 13. Four-Quadrant Synapse Multiply Characteristics vs. InputÊVoltage

# MODE DESCRIPTIONS

## Signature Read

The 80170NX chip contains a signature code to differentiate it from other chips. There are two ways to read the code. When A9 goes to high voltage (12V ± 0.5V), the device is recognized as an Intel component when 89H is read from pins N0-N7 and as an 80170NX chip when CFH is read from pins N8-N15. The other option is to toggle through 16 bits at the NMO pin, reading 89H followed by CFH.

## PDPÑParallel Distributed Processing Mode

Parallel distributed processing is the normal processing mode, described in the Principles of Operation section on page 5 <****replace page number****>. As shown in the Processing Configurations section  on page 7 <****replace page number****>, the input derivation is configuration-dependent, yielding a number of different processing characteristics.

## WMI/WMDÑWeight-Modify Increase and Weight-Modify Decrease Modes

When the LRN pin is set to 1 the chip enters the learning, or training, mode. A synapse is selected by addresses A12-A0 and BIAS. Synapse weights are changed by first measuring the EEPROM-cell threshold (the stored weight) by using the WVTR mode. A calculation is then made of the voltage and pulse width needed for the desired weight modification. By pulsing the calculated high voltages on pins VPP1 and VPP2, the weights are modified one at a time. For details, see the next section, TRAINING.

The weight-setting algorithm applies multiple pulses in the process of setting a weight by using the error observed after each pulse and then making a better estimate for the next pulse. The algorithm also carries what it has learned in the process of setting one weight to the setting of the next weight. This adaptation is important for reducing the number of pulses and hence the total weight-setting time.

## WVTRÑWeight VT Read Mode

A specific floating-gate device storing a desired synapse weight is selected by addresses A12-A0, BIAS, and WT when in WVTR mode. The deviceÕs drain terminal is connected to the SYNO pin by on-chip decoding circuitry to allow VT measurement as shown in Figure 14.

Figure 14. .i.VT Measurement System

As an example, during VT measurement, 3.0 volts is applied to SYNO while monitoring the current into SYNO. VPP1 and VPP2 supply the gate and source voltages, respectively, to the selected floating-gate transistor. VT is measured by setting VPP2 (device source) to 2.0 volts and incrementing VPP1 (the gate) from 0 to 4V until the SYNO (drain) current reaches the specified level (typically 1 to 10 µA). The gate-to-source voltage, VPP1 minus VPP2, that generates this current is the VT. For example, given a 0.25 volt reference EEPROM cell VT, a 0.25 volt weight EEPROM cell VT corresponds to a synapse weight of zero; weight EEPROM cell VT's below 0.25 volt correspond to positive weights. Better synapse multiply characteristics are obtained when a constant common mode is maintained. VT's ranging from Ð1.0 to 1.50 volts are attainable corresponding to a weight range of Ð2.5 to +2.5 volts.

Figure 15 shows synapse multiply characteristics versus floating-gate device VT's at different fixed-input voltages (the inverse of Figure 13).

Figure 15. Synapse Four-Quadrant .i.Multiply Characteristics vs. VT Differential

## RMI, RMD, and RVTRÑReference Modify and Read Modes

These modes are identical to the corresponding weight-related modes WMI, WMD, and WVTR except that they affect the reference EEPROM-cell threshold (VT_REF) instead of the weight EEPROM-cell threshold (VT_WT); see Figure 4.

Zero synapse-weight values occur when VT_WT is set equal to VT_REF. Alternatively, unused synapses can be zeroed by programming both weight and reference cells to maximum (and equal) VT's. This is simpler and faster than setting VT_DIFF precisely to zero, and it reduces power consumption by turning the synapse cells off.

The reference EEPROM cellÕs floating gate can be set at a fixed value or to a value opposite from that of the weight EEPROM cell. For maximum dynamic range of weights, VT_REF should be set opposite to VT_WT, and the average of VT_WT and VT_REF (VT_COM) should be held constant at 0.25 volt, approximately the intrinsic (UV-erased) threshold level, as follows:

and

See Weight Setting in the TRAINING OPERATIONS section on page 26 <****replace page number****> for details.

## SMRÑSum Read Mode

The differential voltage generated at a pair of summing nodes by the sum-of-products differential current can be measured directly in the SMR mode. The differential current is converted to a differential voltage through the use of non-linear resistive loads. The effective resistance of these loads vary with the common-mode current through them (see Figure 16). The common-mode current varies with the number of active synapses in the neuron. Addresses A5-A0 select the desired pair of summing nodes (see Figure 17). The selected summing nodes are switched to the SMO+ and SMOÐ pins. This differential voltage is a physical representation of the total sum of weights*inputs + bias for the selected neuron. This voltage is very small and unbuffered; it may contain systematic offsets and is not intended for external use.

Figure 16. .i.Non-Linear Resistive Summing Load

Figure 17. .i.Summing Nodes Selection and Output

Figure 18(A) shows how a differential voltage would be measured between SMO+ and SMOÐ. This mode may also be used to perturb the selected sum by drawing current either from pins SMO+ or SMOÐ. Drawing current from SMO+ will increase the sum while drawing current from SMOÐ will decrease the sum, which is useful for implementation of the Madaline III learning algorithm.

Figure 18. .i.Summing Node Output Measurement

## SYNRÑSynapse Read Mode

A particular synapse is selected by addresses A12-A0 and BIAS in the SYNR mode, while other synapses connected to the same pair of summing nodes are disabled. This allows measurement of a selected input*weight productÕs contribution to the total sum through the SMO± pins as a function of input level. Although a differential voltage can be measured at the pins SMO±, this is not an accurate representation because of the non-linear load characteristics. It is better to measure actual cell current by forcing both pins to VCC and measuring the difference in SMO+ and SMOÐ current levels, as shown in Figure 18(B). Feedback-Array synapses can be measured by using the input pads in the input multiplex mode. Rows used for threshold setting (a subset of the bias units) always have their inputs set to an internal, constant, positive reference. The effective input varies with VREFi for the Input Array and with VREFo for the Feedback Array.

## NRÑNeuron Read Mode

Addresses A5-A0 select a neuron output to be connected to the NMO pin in NR mode. This allows the neuron outputs to be multiplexed out one at a time by the training system, eliminating the need for multiplexers between layers in a multi-layer network. Figures 18 and 19 show the neuron outputÕs sigmoid transfer characteristics as a function of reference and gain control, versus summing-node current input.

Figure 19. .i.Neuron Output Transfer Characteristic vs. SummingÊCurrentÊandÊVGAIN

Figure 20. .i.Neuron Output Transfer Characteristic vs. SummingÊCurrentÊandÊVREF

The sigmoid has an input offset that varies slightly from chip to chip. The offset can be nulled by using one or more of the bias units. Figure 21 shows the HGAIN sigmoid threshold mode.

Figure 21. .i.Sigmoid Characteristic in VGAIN and HGAIN Modes

## Mode Summary

Table 1 shows the signal states in the various operating modes. Valid BIAS addresses are A9 through A0 (A5-A0 are neuron addresses), with A11 and A10Ê=Ê0 (A11 and A10 are only used for normal synapse selection). In all synapses (external input and bias), A12 selects between Input and Feedback Arrays (inputÊ=Ê0, feedbackÊ=Ê1).

Table 1 .i.Mode Summary

|  | CE# | NE# | HOLD | CLK | RESETI | RESETF | HGAIN | QUER | LRN | SSYN | A0-A5 | A6-A11 | A12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parallel Distributed Processing (PDP): |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDP/64 Inputs | 0 | 0 | X | X | 0 | 1 | 0 | 0 | 0 | 0 | X | X | 0 |
| PDP/128 Inputs | 0 | 0 | 0/1(8) | 1 | 0 | 0 | 0 | 0 | 0 | 0 | X | X | 1 |
| Neuron Disable | 0 | 1 | X | X | 0 | 1 | 0 | 0 | 0 | 0 | X | X | 0 |
| Power Down | 1 | X | X | X | X | X | X | X | X | X | X | X | X |
| Weight Modify: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Wt Mod Increase (WMI) | 0 | 1 | X | X | X(1) | X(1) | X | 0 | 1 | 1 | V | V | V |
| Wt Mod Decrease (WMD) | 0 | 1 | X | X | X(1) | X(1) | X | 0 | 1 | 1 | V | V | V |
| Wt Vt Read (WVTR) | 0 | 1 | X | X | X(1) | X(1) | X | 1 | 1 | 1 | V | V | V |
| Ref Mod Increase (RMI) | 0 | 1 | X | X | X(1) | X(1) | X | 0 | 1 | 1 | V | V | V |
| Ref Mod Decrease (RMD) | 0 | 1 | X | X | X(1) | X(1) | X | 0 | 1 | 1 | V | V | V |
| Ref Vt Read (RVTR) | 0 | 1 | X | X | X(1) | X(1) | X | 1 | 1 | 1 | V | V | V |
| Query: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sum Read (SMR) | 0 | X | V | V | V | V | X | 1 | 0 | 0 | V | X | V |
| Synapse Read (SYNR) | 0 | X | V | V | V | V | X | 1 | 0 | 1 | V | V | V |
| Neuron Read (NR) | 0 | 0 | V | V | V | V | V | 1 | 0 | 0 | V | X | V |
| Reset/Perturb: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Erase Wt Column | 0 | 1 | X | X | X(1) | X(1) | X | 0 | 1 | 0 | V | X | X |
| Erase Ref Column | 0 | 1 | X | X | X(1) | X(1) | X | 0 | 1 | 0 | V | X | X |
| Prog Wt Column | 0 | 1 | X | X | X(1) | X(1) | X | 0 | 1 | 0 | V | X | X |
| Prog Ref Column | 0 | 1 | X | X | X(1) | X(1) | X | 0 | 1 | 0 | V | X | X |
| Sum Read/Perturb (SMR/P) | 0 | 0 | V | V | V | V | V | 1 | 0 | 0 | V | X | V |
| Signature Modes: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Standard | 0 | 0 | X | X | X | X | 0 | 0 | 0 | X | X | X(4) | X |
| NW vs NX | 0 | 0 | X | X | 1 | 1 | 1 | 0 | 0 | X | X | X(4) | X |
| NMO Signature | 0 | 0 | X | X | X | X | 1 | 1 | 0 | X | V | X(4) | X |
| Continuous Feedback: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| With Input Array Disabled | 0 | 0 | X | 1 | 1 | 0 | 0 | 0 | 0 | X | X | X(6) | X |

NOTES:

(1) Preferred RESETI = RESETF = 1, if RESETI/RESETF = 0, the selected synapse will conduct current. This may affect the programming conditions. The state of these pins must be consistent between programming and verifying.

(2) Force SMO± to VCC and measure current.

(3) Sink differential current from the SMO± pins.

(4) A9 = VHH.

(5) VREFo is less than 0.7V.

(6) A11 = VHH.

(7) Outputs may be active, depending on state of NE#.

(8) See Figure 7.

SYMBOLS:

0 = Logic Ò0Ó

0/1 = Transition from 0 to 1 (see Figure 7).

1 = Logic Ò1Ó

X = DonÕt Care Inputs, Indeterminate State Outputs

Q = Valid Data Output

V = Valid Input

Z = High Impedance

VCC = Logic Supply Voltage

VHH = 12 ± 0.5V

VP1 & VP2 = High-Voltage Pulse Inputs

VG & VS = Gate and Source VoltagesÑused in SYNR mode

ID = EEPROM Cell Drain Current

Table 1. Mode Summary (Continued)

|  | BIAS | WT | INC | VGAIN | VREFi | VREFo | I0-I63 | N0-N63 | SMO± | NMO | SYNO | VPP1 | VPP2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parallel Distributed Processing (PDP): |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDP/64 Inputs | X | X | X | V | V | V | V | Q | Z | Z | Z | VCC | VCC |
| PDP/128 Inputs | X | X | X | V | V | V | V | Q | Z | Z | Z | VCC | VCC |
| Neuron Disable | X | X | X | V | V | V | V | Z | Z | Z | Z | VCC | VCC |
| Power Down | X | X | X | X | X | X | X | Z | Z | Z | Z | VCC | VCC |
| Weight Modify: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Wt Mod Increase (WMI) | V | 1 | 1 | X | X | X | X | Z | Z | Z | Z | VP1 | VP2 |
| Wt Mod Decrease (WMD) | V | 1 | 0 | X | X | X | X | Z | Z | Z | Z | VP1 | VP2 |
| Wt Vt Read (WVTR) | V | 1 | X | X | X | X | X | Z | Z | Z | ID | VG | VS |
| Ref Mod Increase (RMI) | V | 0 | 1 | X | X | X | X | Z | Z | Z | Z | VP1 | VP2 |
| Ref Mod Decrease (RMD) | V | 0 | 0 | X | X | X | X | Z | Z | Z | Z | VP1 | VP2 |
| Ref Vt Read (RVTR) | V | 0 | X | X | X | X | X | Z | Z | Z | ID | VG | VS |
| Query: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sum Read (SMR) | X | X | X | X | V | V | V | X(7) | Q | X | Z | VCC | VCC |
| Synapse Read (SYNR) | V | X | X | X | V | V | V | X(7) | VCC(2) | X | Z | VCC | VCC |
| Neuron Read (NR) | X | X | X | V | V | V | V | Q | Q | Q | Z | VCC | VCC |
| Reset/Perturb: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Erase Wt Column | X | 1 | 1 | X | X | X | X | Z | Z | Z | Z | VP1 | VP2 |
| Erase Ref Column | X | 0 | 1 | X | X | X | X | Z | Z | Z | Z | VP1 | VP2 |
| Prog Wt Column | X | 1 | 0 | X | X | X | X | Z | Z | Z | Z | VP1 | VP2 |
| Prog Ref Column | X | 0 | 0 | X | X | X | X | Z | Z | Z | Z | VP1 | VP2 |
| Sum Read/Perturb (SMR/P) | X | X | X | V | V | V | V | Q | Q(3) | Q | Z | VCC | VCC |
| Signature Modes: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Standard | X | X | X | X | X | V | X | Q | Z | Z | Z | VCC | VCC |
| NW vs NX | X | X | X | X | X | V(5) | X | Q | Z | Z | Z | VCC | VCC |
| NMO Signature | X | X | X | X | X | V | X | Q | Q | Q | Z | VCC | VCC |
| Continuous Feedback: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| With Input Array Disabled | X | X | X | V | X | V | X | Q | Z | Z | Z | VCC | VCC |

NOTES:

(1) Preferred RESETI = RESETF = 1, if RESETI/RESETF = 0, the selected synapse will conduct current. This may affect the programming conditions. The state of these pins must be consistent between programming and verifying.

(2) Force SMO± to VCC and measure current.

(3) Sink differential current from the SMO± pins.

(4) A9 = VHH.

(5) VREFo is less than 0.7V.

(6) A11 = VHH.

(7) Outputs may be active, depending on state of NE#.

(8) See Figure 7.

SYMBOLS:

0 = Logic Ò0Ó

0/1 = Transition from 0 to 1 (see Figure 7).

1 = Logic Ò1Ó

X = DonÕt Care Inputs, Indeterminate State Outputs

Q = Valid Data Output

V = Valid Input

Z = High Impedance

VCC = Logic Supply Voltage

VHH = 12 ± 0.5V

VP1 & VP2 = High-Voltage Pulse Inputs

VG & VS = Gate and Source VoltagesÑused in SYNR mode

ID = EEPROM Cell Drain Current

# TRAINING OPERATIONS

## Training Systems

Intel offers a complete development and training system for 80170NX chipsÑthe Intel Neural Network Training System (iNNTS) and the ETANN Multi-chip Board (EMB). The iNNTS and EMB are compatible with any personal computer based on an Intel 80286 (or later) processor with at least 1MB of memory.

The iNNTS includes a Personal Computer PROM Programmer (PCPP), 80170 Adapter, Generic Programmer Interface (GUPI) base, Confidence Test Module, two 80170NX chips, interface software, and two neural network simulation programs.

The simulation programs are iBrainMaker from California Scientific Software, and iDynaMind from NeuroDynamX, Inc. Both simulation programs incorporate the 80170NX chipÕs sigmoid threshold characteristics during simulation. Both programs provide menu-driven functions to download synapse weights to the chip, test through the chip, and perform chip-in-loop optimization to compensate for analog variations. The programs support partitioning of the Feedback Array to allow networks with multiple hidden layers to be implemented on a single chip.

Users who prefer to write their own simulation or training algorithms are provided with a Training System Interface Library (TSIL) containing over two dozen subroutines with which to control the operations of the chip. These routines are described in detail in the ProgrammerÕs Reference section of the iNNTS UserÕs Guide.

## Multi-Chip Training

The 80170 Adapter provides only one socket with which to program and train one chip. Multi-chip designs require the EMB add-on board, which attaches to the iNNTS 80170 Adapter. An EMB includes an eight-socket prototyping board and two 80170NX chips. The iNNTS software is upwardly compatible with the EMB board, and multi-chip versions of the two simulation programs, iBrainMaker and iDynaMind, are provided with the EMB. The TSIL library contains routines to access each socket of the EMB separately. The board is shipped with jumper blocks attached on the bottom side to configure two analog buses for multiplexing 64 analog inputs and outputs to a target socket. When the chips are trained, the user has the option of removing the jumper blocks and wirewraping directly from pin to pin. By connecting one chipÕs output pins directly to another chipÕs input pins, real-time processing  configurations can be prototyped.

## Chip-In-Loop Training

Due to fabrication variations, the characteristics of synapses and neurons vary within a single chip and within batches of chips. To compensate for these variations and to optimize chip performance, both the iNNTS and the EMB support chip-in-loop training.

This method replaces the synapse weights that would otherwise be simulated by a simulation program, with actual weights stored in a chip. During training, weights are downloaded to the chip and outputs of the chip are fed back to the training simulator, which evaluates the result and updates the chipÕs weights accordingly.

## Weight Setting

Changes to synapse weights are accomplished by applying high-voltage pulses in the WMI and WMD modes. First, one of the two floating-gate devices in a single synapse is selected by the addresses A12-A0, BIAS, and WT, then high-voltage pulses are applied on pins VPP1 and VPP2 (see Figure 32). An adaptive algorithm is used to give a precise target weight with a minimum number of pulses, independent of synapse-to-synapse variations.

The EEPROM cellÕs floating-gate charge is modified by applying a high-voltage pulse for tens of microseconds. The weight-setting algorithm determines the pulse voltage, VPP2. After the pulse is applied, the algorithm reads the synapse weight. If there are differences from the target weight, a subsequent pulse is applied.

The synapse weight is directly related to the difference in threshold voltages of the two floating-gate devices in that synapse (see VT Read mode description). Floating-gate threshold voltages are measured repeatedly during the weight-setting algorithm to assess how well the weight-change process is proceeding and to make better estimates of the voltage to be used in subsequent weight settings.

The difference between target VT (VT_target) and real VT (VTeo) is ÆVT. The floating gate is programmed when ÆVT is positive and erased when ÆVT is negative. In other words, programming raises the EEPROM cell threshold, while erasing reduces its threshold:

ÆVTÊ=ÊVT_target ÐÊVTeo
	If ÆVT > 0,Êthen program
	If ÆVT < 0,Êthen erase

Figures 21 and 22 illustrate the calculated relationship between a desired synapse-weight change, ÆVT, and the high voltage which should be applied at VPP2 to achieve it. A family of calculated curves is shown for various initial thresholds, VTeo, for a fixed pulse width of 100 µs to illustrate the relationship. The exact relationship may vary slightly from these values. Table 2 shows the equations from which the curves were generated.

Figure 22. .i.VPP2 Voltage Curves for Positive ÆVT Ñ Program

Figure 23. .i.VPP2 Voltage Curves for Negative ÆVT Ñ Erase

Table 2. .i.Training/Weight Setting Algorithm Equations

Figure 24 is a flow chart of the weight-setting algorithm. The algorithm begins by initializing the address of the floating-gate device and two coefficients: BPGM and BERS (BPGM for programming, BERS for erase). The ÆVT for the next iteration is determined from the difference between the VT_target and the most recently measured VT (=ÊVTeoÊ=ÊVT_real). The VPP2 for the next pulse is iteratively solved for by using the functions given in Table 3, f(VPP_old) or g(VPP_old), for programming and erasing, respectively. The functions fÕ and gÕ signify the derivatives of these functions, respectively.

Figure 24. .i.Weight-Setting Algorithm

After applying a pulse, the threshold is measured again and subtracted from the original threshold to arrive at the real ÆVT. If the measured ÆVT deviates by more than 1% from the desired ÆVT, a new BPGM or BERS will be calculated based on the previous VPP and BPGM or BERS. This procedure is repeated until the measured VT falls within 1% of the VT_target. The final BPGM and BERS will be stored for use by the next synapse in the same chip. Because synapses on a chip have very similar program and erase characteristics, the B coefficients do not change significantly after the first few synapse weights are modified.

The difference between the desired ÆVT and the actual ÆVT is reduced by each subsequent pulse as the adaptive algorithm learns from experience.

## Madaline Learning

Using SMO+ and SMOÐ in the SMR/P (Sum Read/Perturb) mode allows adding or subtracting incremental differential current from a selected neuronÕs summing nodes. Sum perturbation supports training by the Madaline learning algorithm. This involves the application of many perturbations followed by grading the improvements produced by each perturbation. Synapse weights are then changed to produce the same effects as the perturbations which caused the most significant improvements in the output responses.

## Function Simulation

Many standard neural network simulators can model the 80170NX chipÕs dot product neurons reasonably well. The constraints of the chipÕs analog circuits can be reflected by specific limits on input and weight-setting ranges and gain values. A first-order emulation allows evaluation of whether a chip or group of chips can be trained to perform the desired pattern-mapping application. Currently there are no effective analytical techniques or algorithms for determining appropriate neural network architectures by simply observing the training information set.

The following transfer characteristic simulates the chipÕs neuron output at typical operating conditions (VGAIN = 5V, VREFo = 1.4V, VREFi = 1.4V, 64 input operation):

vjÊ=Ê{2 / (1 + e-8·ui*Wij)} Ð 1

where:

ui	Inputs, constrained so that Ð1 ² ((VjÐVREFi)/VREFi) ² +1.

Wij 	Weights, constrained so that Ð1 ² ((weight VTÐreference VT)/2.5V) ² +1.

vj	Outputs, constrained so that Ð1 ² ((OutputÐVREFo)/VREFo) ² +1.

These normalized input and synapse-weight values correspond to actual inputs of 0 to 2.8V and weights of Ð2.5 to 2.5V. The normalized output will also correspond to an actual range of 0 to 2.8V.

A more accurate model of the chipÕs output under the above conditions and with VGAIN = 5V is:

vjÊ= {1.8 Ú (1 + eÐ8(·iui(1.2Ð0.2ui2)Wij(1.5-0.5Wij2)Ð·kBkj } Ð 0.9

This model includes the roll-off in the synapse characteristic at large weight and input magnitudes, as well as the output range limitations. The estimated equation for VGAIN = 3.3V (with VREFi = VREFi = 1.5V) is:

## Bake-Train-Bake

The chipÕs weight retention over extended periods of time can be significantly improved by a process called Bake-Train-Bake. Charge movement in the nitride layer between the EEPROM cellÕs control gate and floating gate creates dipoles. The dipoles compensate the electric field created by charges on the floating gates, thus affecting the effective floating-gate potential. This electrical reaction offsets the effective VT_DIFFÕs from their original targets; thus, affecting the chipÕs processing accuracy. High-temperature bake operations accelerate this phenomenon. When combined with re-training, the net effect is to reduce this VT shift, since the electrical field differences between re-trainings is much smaller than between the originally untrained and the first-training conditions. The change in synapse weight versus bake time is shown in Figure 25. The improvement in a single 24-hour 250¡C bake-train operation represents 3 to 4 bits of resolution over the chipÕs minimum data-retention lifetime of 10 years. Two bake-train cycles compensate for lifetime level VT shifts well within the chipÕs intrinsic processing accuracy. The Acceleration Factor note at the top-left of the chart indicates that baking at 260¡ C causes the percent change shown on the vertical axis to occur 1000 times faster than it would occur if the chip were stored at 125¡ C, and 100000 times faster than it would occur if the chip were stored at 75¡ C.

Figure 25. .i.Reduction of VT Shifts Through Bake-Train-Bake

## Temperature and VCC Performance

Figures 25 and 26 show a typical neuronÕs response to ambient temperature and VCC, respectively. For typical input patterns and synapse weights, four inputs at Weight = 0.5V (VT_DIFF) are swept while monitoring the neuron output at VGAIN=5V.

Figure 26. Sigmoid .i.Temperature Sensitivity

Figure 27. Sigmoid .i.VCC Sensitivity

In Figure 26, two temperature effects are visible in the high-gain region of the response: the offset of the neuron shifts and the slope (effective gain) of the neuron decreases. The offset change corresponds to a change in the sum of products of about 2 µA or about 1/5 of one synapse at full weight and full input (0.25% out of 64 synapses) over the commercial temperature range. The offset change is highly dependent on the inputs and weights employed. The output slope decreases with temperature by 0.3% per degree Celsius over the nominal at 25¡C (total 20% change over the commercial range).

In Figure 27, the slope in the high-gain region of the sigmoid characteristic does not change significantly with VCC. The VCC effect is characterized as almost exclusively as a change in offset. The offset shifts by about 0.26V which corresponds to less than 2.5 µA in the total sum over the 4.75V to 5.25V VCC range (±5%).

For both the temperature and VCC cases, the voltage change at the saturated regions of the (extremes of low and high inputs) is less than 10 mV.

# performance

## Weight-Cycling Performance

EEPROM cycling failures are a common concern. The strong electric field required by thin oxide EEPROMs for tunneling can tear the oxide in defect regions. Several process and design improvements have been introduced to combat the problem.

IntelÕs ETOX-II flash memory technology is designed for extended cycling. An advanced tunnel oxide increases charge carrying capability ten-fold, minimizing the oxide area per synapse subjected to the tunneling electric field, and consequently reducing the probability of encountering an oxide defect.

During normal operation with the iNNTS training system, programming and erasing is performed by the weight-setting algorithm described above (see TRAINING OPERATIONS on page 26 <****replace page number****>). Programming and erasing synapse weights is accomplished by cycling the transistors of the reference EEPROM cells and weight EEPROM cells. The 80170NX chip is specified for a minimum of 10,000 such cycles. Chip-cycling performance was verified by performing cycling experiments on 50 chips. A cycle was defined as first setting the weight to Ð2.5V (or less) and then setting it to +2.5V (or more). All 10,240 synapses of each chip were subjected to 10,000 cycles. After the experiment, the iNNTS still had no difficulty setting all weights to ±2.5V.

## Performance After Radiation Exposure

The 80170NX chipÕs behavior and recognition performance were evaluated after subjecting the chip to ionization radiation (see Castro, et al, 1992). The chipÕs were irradiated with electrons at 10MeV at a flux density between 1.8x1010/cm2 and 4.1x1011/cm2 up to an accumulated dose of about 26krad. Two pattern-recognition networks were involved in the study: a 64-45-52 character-recognition network and an 80-54-10 digit-recognition network.

Significant degradation in recognition performance occurred after about 6krad of irradiation. Figure 28 illustrates the mean-square-error (MSE) and misclassification statistics for the character-recognition network (for 104 patterns) as a function of radiation dose and elapsed time for one chip.

The degradation in performance can be attributed in part to the relaxation of the synapse weights shown for the same chip in Figure 29. The relaxation of synapse weights results from the combined effects of oxide charge generation and trapping, and the photovoltaic effect caused by Brehmstralung radiation. Figures 28 and 29 show that the relaxation of weights continues after irradiation and has the most dominant effect on the degradation of recognition performance.

The chip-in-loop training method was employed on one of the character-recognition chips after each exposure to irradiation. 100% recognition was achieved within two  such chip-in-loop training sessions, up until the last exposure at the accumulated dose of 26krad. Apparently, some permanent damage to the synapse weights occurred at that dose. Nevertheless, none of the irradiated chips exhibited total malfunction in performing classifications. Despite the failure of some synapses, most irradiated chips can be re-trained with the chip-in-loop method and by re-locating the weight array. However, the level of degradation in classification performance and the ability to be re-trained via chip-in-loop training are dependent on the particular network architecture, synapse weights, and reserve capacity of the synapse weights.

Figure 28. Misclassified Patterns and MSE After .i.Radiation Exposure

Figure 29. Weight Distributions After .i.Radiation Exposure

# ABSOLUTE MAXIMUM RATINGS

Operating Temperature	0¡C to +70¡C
Storage Temperature	Ð65¡C to +125¡C
Bake-Train-Bake Temperature	75¡C to +260¡C
Voltage on Digital Inputs
Êwith Respect to VSS	Ð2.0V to +7.0V(1)
Voltage on Analog Inputs and
ÊOutputs; with Respect to VSS	Ð2.0V to +7.0V(1)
VCC Supply Voltage
Êwith Respect to VSS	 Ð2.0V to +7.0V(1)
VPP1, VPP2 Voltages
Êwith Respect to VSS	 Ð2.0V to +21.0V(1)
Output Short Circuit Current	 100 mA(2)

* NOTICE: Stresses above those listed under Absolute Maximum Ratings may cause permanent damage to the device. This is a stress rating only and functional operation of the device at these or any conditions above those indicated in the operational sections of this specification is not implied. Exposure to Absolute Maximum Rating conditions for extended periods may affect device stability.

All specifications contained within the following tables are subject to change.

NOTES:
1. Minimum D.C. input voltage is Ð0.5V. During transitions: digital and analog inputs may undershoot to Ð2.0V; VPP supplies may overshoot to +21.0V; and analog inputs and outputs may overshoot to voltages of VCC + 2.0V, but for time periods of only 20Êns or less.

2. Output Short Circuit is for no more than 1 second, with no more than 1 output shorted at a time.

# OPERATING CONDITIONS

| Symbol | Parameter | Min | Max | Units | Conditions |
| --- | --- | --- | --- | --- | --- |
| TA | Operating Temperature | 0 | 70 | ¡C | Processing and Training |
| VCC | VCC Supply Voltage | 4.75 | 5.25 | V |  |

# DC CHARACTERISTICS

At Room Temperature

| Symbol | Parameter | Min | Max | Units | Conditions |
| --- | --- | --- | --- | --- | --- |
| ILI | Input Leakage Current |  | ±0.1 | µA | VIN=3.5V |
| ILO | Output Leakage Current |  | ±1.0 | µA | VOUT=3.5V |
| IOH | Output High Source Current |  | Ð400 | µA | VOUTÊ³Ê2.4V, VREFo=1.4V |
| IOL | Output Low Sink Current |  | 2.1 | mA | VOUTÊ²Ê.45V, VREFo=1.4V |
| VIA | Analog Input Voltage | 0 | 3.5 | V | VCC=5.0V |
| VOA | Analog Output Voltage | 0 | 4.0 | V | VCC=5.0V, RL=10k½ |
| ICC | VCC Active Supply Current |  | 450 | mA | VCC=5.0V, CE#=VIL |
| ICCS | VCC Standby Supply Current |  | 1 | mA | VCC=5.0V, CE#=VIH |
| IPI | VPP1 Programming Supply Current |  | 1 | mA | VPP1=20.0V, VCC=5.0V |
| IP2 | VPP2 Programming Supply Current |  | 1 | mA | VPP2=18.0V, VCC=5.0V |
| VP1 | VPP1 High-Voltage Switch Voltage | 18 | 19 | V | see Training Algorithm |
| VP2 | VPP2 Weight Modify Pulse Voltage | 12.5 | 18 | V | see Training Algorithm |
| VPL | VPP1, VPP2 Inactive/Low Voltage | 4.75 | 5.25 | V |  |
| VIL | Digital Input Low Voltage | Ð0.1 | 0.8 | V | VCC=5.0V |
| VIH | Digital Input High Voltage | 2.0 | 5.5 | V | VCC=5.0V |
| VREFi | Input Reference Voltage | 0 | 1.7 | V | [VREFi(TTL)=1.4V] |
| VREFo | Output Reference Voltage | 0.5 | 2.0 | V | For Symmetric operation maximum VREFo(TTL)= 1.6V |
| VGAIN | Gain Control Voltage | 0.0 | 5.0 | V |  |
| VT_WT, VT_REF | Threshold Voltage (VT) of weight EEPROM cell or reference EEPROM cell | Ð1.0 | 1.5 | V | ID=10ÊµAÊ@ÊSYNO VS=2.0VÊ@ÊVPP2 |
| VT_COM | VTÊCommonÊMode = (VT_WTÊ+ÊVT_REF)/2 | 0 | 0.5 | V | VT_COM typical =0.25V |
| VT_DIFF | VTÊDifference = (VT_WTÊÐÊVT_REF), WeightÊDynamicÊRange | Ð2.5 | 2.5 | V |  |
| Weight | Weight = - VT_DIFF |  |  |  |  |
| See Figure 27 for power supply sensitivity. |  |  |  |  |  |

# AC CHARACTERISTICS

| Symbol | Parameter | Min | Max | Units | Conditions |
| --- | --- | --- | --- | --- | --- |
| tIVQV | Processing Delay (TP) VGAIN HGAIN |  | 3 1.5 | µs µs | with 6- to 7-bit resolution at output pin, ZI=100 pf and 10k½ to ground VREFo=2V |
| tELQV | Chip Enable to Output Valid (TCE) |  | 20 | µs |  |
| tNLQV | Neuron Enable to Output Valid (TNE) |  | 5 | µs |  |
| tRHQV | RESETI/f High/Low to Valid Output |  | 5 | µs |  |
| tIVHH | Input Valid to HOLD High Setup | 150 |  | ns |  |
| tCL-H/CH-L | CLK Input Rise and Fall Time |  | 100 | ns |  |
| tCHCL | CLK High Pulse Width | 150 |  | ns |  |
| tQVCH | Output Valid (Feedback Input) to CLK High Setup | 150 |  | ns |  |
| tIH | CLK Overlap for Reset | 150 |  | ns |  |
| VfO | Output Slew Rate | 5 |  | V/µs | CLÊ=Ê100 pF |
| tHOLD | Sample and Hold Circuit Hold Time |  | 10 0.8 | s s | 1% droop @ TAÊ=Ê25¡C 1% droop @ TAÊ=Ê70¡C |
| tIVP1H | Input Valid to VPP1 Setup | 200 |  | ns |  |
| tP1HP2H | VPP1 Setup Time Before VPP2 | 100 |  | ns |  |
| tP2L-H/H-L | VPP2 Pulse Rise/Fall Time | 500 |  | ns |  |
| tP2HP2L | VPP2 Pulse Width | 0.5 | 1000 | µs |  |
| tPLIV | VPP1, VPP2 Recovery Time | 10 |  | µs |  |

# CAPACITANCE

| Symbol | Parameter | Min | Max | Units | Conditions |
| --- | --- | --- | --- | --- | --- |
| CIN | Input Capacitance |  | 12 | pF |  |
| COUT | Output Capacitance |  | 12 | pF |  |

Figure 30. .i.AC Waveforms for Parallel Processing

Figure 31. .i.AC Waveforms for QUERY Modes

Figure 32. .i.AC Waveforms for Weight Modify

Figure 33. 208-Pin PGA .i.Package

# Pin Assignments

| A1 - N14 A2 - N.C. A3 - N17 A4 - N20 A5 - VCC A6 - N21 A7 - N25 A8 - N29 A9 - VCC A10 - N34 A11 - N38 A12 - N42 A13 - VCC A14 - N43 A15 - N46 A16 - N.C. A17 - N49 B1 - N10 B2 - N15 B3 - N16 B4 - N18 B5 - N.C. B6 - N24 B7 - N23 B8 - N27 B9 - N31 B10 - N36 B11 - N40 B12 - N39 B13 - N.C. B14 - N45 B15 - N47 B16 - N48 B17 - N53 | C1 - N8 C2 - N13 C3 - N12 C4 - N.C. C5 - N19 C6 - VSS C7 - N26 C8 - N30 C9 - N32 C10 - N33 C11 - N37 C12 - VSS C13 - N44 C14 - N.C. C15 - N51 C16 - N50 C17 - N55 D1 - N6 D2 - N9 D3 - N.C. D4 - N.C. D5 - N.C. D6 - N.C. D7 - N22 D8 - N28 D9 - VCC D10 - N35 D11 - N41 D12 - N.C. D13 - N.C. D14 - N.C. D15 - N.C. D16 - N54 D17 - N57 | E1 - N4 E2 - N5 E3 - N.C. E4 - N11 E14 - N52 E15 - N.C. E16 - N58 E17 - N59 F1 - N2 F2 - HGAIN F3 - N3 F4 - N7 F14 - N56 F15 - N60 F16 - D.U. F17 - N61 G1 - VGAIN G2 - N0 G3 - VREFo G4 - N1 G14 - N62 G15 - SMOÐ G16 - N63 G17 - NMO H1 - VCC H2 - VREFi H3 - RESETF H4 - VSS H14 - VSS H15 - A0 H16 - SMO+ H17 - VCC J1 - I63 J2 - CLK J3 - HOLD J4 - RESETI | J14 - A2 J15 - A3 J16 - A1 J17 - A4 K1 - I61 K2 - I59 K3 - I62 K4 - I60 K14 - A7 K15 - A5 K16 - A8 K17 - A6 L1 - I57 L2 - I55 L3 - I58 L4 - I54 L14 - BIAS L15 - A9 L16 - A12 L17 - A10 M1 - I53 M2 - I56 M3 - I52 M4 - I48 M14 - LRN M15 - NE# M16 - A11 M17 - WT N1 - I51 N2 - I50 N3 - N.C. N4 - I44 N14 - SSYN N15 - N.C. N16 - QUER N17 - SYNO | P1 - I49 P2 - I46 P3 - N.C. P4 - N.C. P5 - N.C. P6 - N.C. P7 - I31 P8 - I25 P9 - VSS P10 - I18 P11 - I12 P12 - N.C. P13 - N.C. P14 - N.C. P15 - N.C. P16 - VPP2 P17 - INC Q1 - I47 Q2 - I42 Q3 - I43 Q4 - N.C. Q5 - I36 Q6 - I33 Q7 - I27 Q8 - I23 Q9 - I21 Q10 - I20 Q11 - I16 Q12 - I10 Q13 - I7 Q14 - N.C. Q15 - I0 Q16 - I1 Q17 - CE# | R1 - I45 R2 - I40 R3 - I39 R4 - I37 R5 - N.C. R6 - I29 R7 - I30 R8 - I26 R9 - I22 R10 - I17 R11 - I13 R12 - I14 R13 - N.C. R14 - I6 R15 - I4 R16 - I3 R17 - VPP1 S1 - I41 S2 - N.C. S3 - I38 S4 - I35 S5 - I34 S6 - I32 S7 - I28 S8 - I24 S9 - VSS S10 - I19 S11 - I15 S12 - I11 S13 - I9 S14 - I8 S15 - I5 S16 - N.C. S17 - I2 |
| --- | --- | --- | --- | --- | --- |

N.C.Ê=ÊNO CONNECT	D.U.Ê=ÊDON'T USE (RESERVED)

# ORDERING INFORMATION

| Test/Temperature Category | Package Type | Performance Specification | Order Code |
| --- | --- | --- | --- |
| Commercial | 208-PGA | Standard-tIVQVÊ=Ê3 µS | A80170NX |

# REFERENCES

## General

80170NX Neural Network Technology and Applications. Intel Corp., 1992.

Holler, M., S. Tam, H. Castro, and R. Benson. ÒAn Electrically Trainable Artificial Neural Network (ETANN) with 10240 ÔFloating GateÕ Synapses," International Joint Conference on Neural Networks, June 1989. Vol. II, 191.

Roy, M., M. Holler, S. Tam, and H. Castro. ÒTesting An Analog VLSI Neural Network with 10240 ÔFloating GateÕ Synapses," 1990 IEEE VLSI Test Symposium.

Tam, S., M. Holler, J. Brauch, A. Pine, A. Peterson, S. Anderson, and S. Deiss. ÒAÊReconfigurable Multi-Chip Analog Neural Network; Recognition and Back Propagation Training," 1992 International Joint Conference on Neural Networks, Baltimore, Maryland. Vol. 2, 625.

Tam, S., B. Gupta, H. Castro, and M. Holler. ÒLearning On An Analog VLSI Neural Network Chip," Proceedings of the 1990 IEEE International Conference on Systems, Man, and Cybernetics. November 1990, pp. 701-703.

Calvin, J., S. Rogers, D. Zahirniak, ÒCharacterization of the ETANN Chip sigmoidal Transfer Function for a System VGAIN = 3.3VÓ (available from Intel).

Castro, H., et al, ÒRadiation Exposure Effects on the Performance of the ETANN (80170NX)," Intel Corporation, 1992.

Kern, L., D. Andes, ÒDesign and Development of a Real-Time Neural Processor Using the Intel 80170NX Electrically Trainable Artificial Neural Network (ETANN)," 1992 Government Microcircuit Application Conference, Digest of Papers, pp. 269-272.

Badgett, W., K. Burkett, M. Campbell, D. Wu, B. Denby, C. Lindsey, R. Blair, S. Kuhlmann, J. Romano, ÒA Neural Network Calorimeter Trigger Used in CDF,Ó IEEE 1992 Nuclear Science Symposium and Medical Imaging Conference, October 25-31, 1992, Orlando, Florida.

Thorson, M., F. Warthman, M. Holler, ÒA Neural-Network Audio Synthesizer," Dr. DobbÕs Journal, February, 1993.

## Cognitive Psychology

Rumelhart, D.E. and J.L. McClelland. Parallel Distributed Processing Explorations in the Microstructure of Cognition.
ÊÊVolume 1: Foundations
ÊÊVolume 2: Psychological and Biological Models
MIT Press, Bradford Book, Cambridge, Mass, 1988. Original printing 1986.

Rumelhart, D.E. and J.L. McClelland. Explorations in Parallel Distributed Processing: A Handbook of Models, Programs, and Exercises. (Programs on floppy will run on an IBM PC. C Source code included). MIT Press, Bradford Book, Cambridge, Mass., 1988.

Rumelhart, David, Geoffrey E. Hinton, and Ronald J. Williams. ÒLearning Representations by Back-Propagating Errors," Nature Vol. 323 No. 9, October 1986.

## Neurobiology

Kandel, E. R., and J. H. Schwartz. Principles of Neural Science. 2nd edition. Elsevier, New York. Medical text with extensive detail on the function of the neuron and the human nervous system.

Burnod, Y. An Adaptive Neural Network: The Cerebral Cortex. Prentice Hall, London, 1990.

## Neural Network Models and Learning

Kohonen, T. Self-Organization and Associative Memory. Springer-Verlag, New York. 2nd edition 1988. First edition 1984. Good general reference.

Lippmann, R. P. ÒAn Introduction to Computing with Neural Nets," IEEE ASSP Magazine, April 1987. Overview article of the various networks and learning paradigms for an issue of ASSP dedicated to neural networks.

Koch, C. and I. Jegev, ed. Methods in Neuronal Modeling. The MIT Press, Cambridge, Mass., 1989.

Hinton, G., and J. Anderson, ed. Parallel Models of Associative Memory. Lawrence Erlbaum Associates, Publishers, Hillsdale, New Jersey, 1989.

Kanerva, P. Sparse Distributed Memory. Bradford Book, The MIT Press, Cambridge, Mass., 1988.

Hopfield J. J. and D. W. Tank. ÒComputing with Neural Circuits: A Model," Science Vol. 233, August 1986: 625.

Anderson, J. and E. Rosenfeld, ed. Neurocomputing Foundations of Research. The MIT Press, Cambridge, Mass., 1988.

Andes, D., B. Widrow, M. Lehr, E. Wan. ÒMadaline Rule III: A Robust Algorithm for Training Neural Networks," International Joint Conference on Neural Networks, 1990, Vol. I: 533-536.

## Speech Recognition

Parsons, T. Voice and Speech Processing. McGraw-Hill, New York.

Waibel, A., T. Hanazawa, G. Hinton, K. Shikano, and K. Lang. ÒPhoneme Recognition Using Time-Delay Neural Networks," IEEE Trans. on Acoustics, Speech and Signal Processing Vol. 37, No. 3, March 1989. Work by Japanese Research consortium ATR in conjunction with Carnegie-Mellon University.

Waibel, A. and K. Lee, ed. Readings in Speech Recognition. Morgan Kaufmann Publishers Inc., San Mateo, California, 1990.

## Temporal Pattern Recognition

Wolf, L. Recurrent Nets for the Storage of Cyclic Sequences.

Williams, R. J. and D. Zipser. ÒA Learning Algorithm For Continually Running Fully Recurrent Neural Networks," Institute for Cognitive Science Report #8805, Oct. 1988, U.C. San Diego.

Pearlmutter, B. A. ÒLearning State Space Trajectories in Recurrent Neural Networks," Proceedings of the International Joint Conference on Neural Networks, June 1989, Vol. II: 365.

## Implementation

Mead, C. Analog VLSI and Neural Systems. Addison Wesley, New York, 1989.

Mead, C. and M. Ismail, ed. Analog VLSI Implementation of Neural Systems. Kluwer Academic Press, Boston, 1989.

von Neumann, J. The Computer and the Brain. Yale University Press, New Haven, Connecticut, 1958.

## Control

Nguyen, D. and B. Widrow. ÒThe Truck Backer Upper: An Example of Self Learning in Neural Networks," International Joint Conference on Neural Networks, June 1989, Washington, D.C.

Miller, W., R. Sutton, P. Werbos. ÒNeural Networks for Control," The MIT Press, Cambridge MA, 1991.

Staib, W. and R. Staib. ÒThe Intelligent Arc Furnace Controller: A Neural Network Electrode Position Optimization System for the Electric Arc Furnace," International Joint Conference on Neural Networks, June 1992, Vol. III: 1.

# Glossary

Bias SynapseÑSee fixed-input bias synapse.

Bake-Train-BakeÑAn experimental procedure used to enhance the long-term stability of the 80170NX synapse weights. In this procedure, the chip is baked, then trained, then baked again.

Chip-In-Loop TrainingÑA procedure used to train an 80170NX-based neural network, also know as chip-in-loop optimization. In this procedure, actual 80170NX neuron outputs are used in the training algorithm to arrive at the weight updates. The procedure compensates for non-ideal characteristics of the chip and the training algorithm.

Dot ProductÑThe quantity obtained by multiplying the components of two vectors (the external inputs and their corresponding synapse weights) and adding the products. Also called inner product or scalar product. The equation is:

EraseÑTo decrease the threshold voltage of a weight EEPROM cell or reference EEPROM cell.

EEPROMÑElectrically erasable programmable read-only memory.

EEPROM CellÑOne of two floating-gate EEPROM cells that, together, store a synapse weight. The two cells are the weight EEPROM cell and the reference EEPROM cell. The synapse weight is the difference between the threshold voltages at each of these cells.

External InputÑOne of 64 input pins on the 80170NX chip.

External-Input SynapseÑA synapse connected to an external input.

Feedforward Neural NetworkÑA neural network in which inputs are received externally or from a previous layer, are processed, and are passed on to only  one subsequent layer. Neurons in a given layer do not connect to one another.

Fixed-Input Bias SynapseÑA synapse used to provide the threshold for the neuronÕs sigmoid function and to cancel offsets due to physical non-linearities in the chip. The Input and Feedback Arrays each have 16 fixed-input bias synapses. Mathematically, all of these bias synapses act as a single bias. However, since the dynamic range of a single EEPROM bias synapse is limited, 16 of them are used. All 16 contribute to the dot product performed by the neuron, although seven are set by the training program and the other nine are set by a chip-initialization process.

Hidden LayerÑA group of neurons that is not connected directly to output pins on the chip. Any of the layers in a feedforward neural network that processes information prior to the output layer.

Inner ProductÑSee dot product.

NeuronÑIn biology, a cell that produces an output signal in response to multiple input signals. In the 80170NX chip, a structure that sums the dot products of input signals and synapse weights, then computes a sigmoid threshold function to produce an output signal.

Output LayerÑA group of neurons with outputs that are externally accessible.

PatternÑAn input vector. The 80170NX can accept input vectors with up to 128 components.

PDPÑParallel distributed processing. A term for neural network processing. In PDP, a particular piece of information is represented by a distributed pattern of activity on a number of neurons rather than by the activity of a single neuron. This method of distributed representation is the source of a neural networkÕs robustness to faults in individual neurons. The term was coined by Rumelhart, et. al.

ProgramÑTo increase the threshold voltage of a weight EEPROM cell or reference EEPROM cell.

Reference EEPROM CellÑOne of two floating-gate EEPROM cells that, together, store a synapse weight. See EEPROM cell and weight EEPROM cell.

SynapseÑIn biology, a variable-strength connection between neurons that can be either output-exciting or output-inhibiting. In the 80170NX chip, a structure that includes a multiplier and a stored weight. The synapse multiplies an input by the weight stored in the synapse and contributes this product to the dot product calculated by the neuron.

Sigmoid FunctionÑA monotonically increasing function that is shaped like a stretched-out letter ÒS.Ó This nonlinear function gives the neuron the ability to make ÒdecisionsÓ and perform non-linearly separable classification tasks.

Synapse WeightÑA non-volatile weight that is stored at a synapse. The value of the weight can be between between +2.5 and -2.5.

Two-Layer Neural NetworkÑA neural network consisting of one hidden layer and one output layer.

WeightÑSee synapse weight.

Weight EEPROM CellÑOne of two floating-gate EEPROM cells that, together, store a synapse weight. See EEPROM cell and reference EEPROM cell.

Weight SettingÑThe process of programming one EEPROM cell and erasing another EEPROM cell, in a synapse cell-pair, so as to set a desired weight value for that synapse.

# Index

128-Input Feedback Configuration, 13

128-Input Feedforward Configurations, 10

64-Input Feedback Configurations, 11

64-Input Feedforward Configurations, 9

64-Input Sequence Recognition and Generation Configurations, 12

80170 Adapter, 26

A0-A5, 24

A11, 11

A12, 3, 8, 9, 11, 24

A12-A0, 3, 16, 17, 21, 26

A5-A0, 19, 21

A6-A11, 24

A9, 16

ABSOLUTE MAXIMUM RATINGS, 36

AC CHARACTERISTICS, 38

AC Waveforms for Parallel Processing, 39

AC Waveforms for QUERY Modes, 39

AC Waveforms for Weight Modify, 40

Acceleration Factor, 31

ADDRESSES, 3

ambient temperature, 32

Analog Input Voltage, 37

Analog Output Voltage, 37

Applications, 1

Architecture, 1

Bake-Train-Bake, 31, 45

Bake-Train-Bake Temperature, 36

BERS, 29

BIAS, 4, 16, 17, 21, 26

bias array, 1

Bias Synapse, 45

BIAS-SYNAPSE SELECT, 4

bias-synapse weights, 7

Bkj, 5

Block Diagram, 2

BPGM, 29

Brehmstralung radiation, 34

bus interconnection, 13

California Scientific Software, 26

CAPACITANCE, 38

Charge movement, 31

chip architecture, 5

CHIP ENABLE, 3

Chip Enable to Output Valid, 38

Chip-cycling performance, 34

Chip-In-Loop Training, 26, 34, 45

CHMOS*III, 1

CIN, 38

CLK, 3, 8, 9, 10, 11, 13, 24

CLK High Pulse Width, 38

CLK Input Rise and Fall Time, 38

CLK Overlap for Reset, 38

common-mode current, 19

Confidence Test Module, 26

Connections Per Second, 1

Continuous Feedback, 24

control-gate voltage, 15

COUT, 38

CPS, 1

current-voltage characteristics, 15

cycles, 34

cycling failures, 33

DC CHARACTERISTICS, 37

development and training system, 26

differential EEPROM cell pairs, 6

differential voltage, 19

Digital Input High Voltage, 37

Digital Input Low Voltage, 37

digital interfacing, 1

dipoles, 31

dot product, 1, 5, 45

EEPROM, 45

EEPROM Cell, 45

EEPROM cell pairs, 6

EEPROM cells, 15

EEPROM Floating-Gate Cell Characteristics, 15

EEPROM technology, 1

EMB, 13, 26

Erase, 45

Erase Ref Column, 24

Erase Wt Column, 24

ETANN, 1

ETANN Multi-chip Board (EMB), 13, 26

ETOX-II flash memory technology, 33

External Input, 45

External-Input Synapse, 45

Feedback Array, 1, 5

FEEDBACK INPUTS, 3

Feedback-Array processing, 10

feedforward configuration, 8

Feedforward Neural Network, 45

Fixed-Input Bias Synapse, 45

fixed-input bias synapses, 5, 6

floating-gate cells, 15

floating-gate potential, 31

floating-gate thresholds, 7

floating-gate voltage, 7

four-quadrant multiply characteristics, 16

Fowler-Nordheim tunneling, 15

Function Simulation, 30

GAIN CONTROL VOLTAGE, 3, 37

Generic Programmer Interface, 26

Glossary, 45

GUPI, 26

HGAIN, 1, 3, 22, 24

Hidden Layer, 45

HIGH GAIN MODE, 3

high voltage, 15

High-temperature bake, 31

high-voltage pulses, 26

HOLD, 3, 7, 9, 13, 24

HOLD INPUT, 3

Hopfield networks, 11

I0-I63, 3, 8, 9

iBrainMaker, 26

ICC, 37

ICCS, 37

iDynaMind, 26

ILI, 37

ILO, 37

INC, 4

INCREASE or DECREASE SYNAPSE WEIGHT, 4

inner product, 1, 45

iNNTS, 26

INPUT AND OUTPUT REFERENCE VOLTAGES, 3

Input Array, 1, 5

Input Capacitance, 38

Input Leakage Current, 37

input multiplex circuits, 8

INPUT MULTIPLEXER, 3

Input Multiplexing Circuits, 8

Input Reference Voltage, 37

Input Valid to HOLD High Setup, 38

Input Valid to VPP1 Setup, 38

input vectors, 8

inputs, 1, 5

Intel Neural Network Training System (iNNTS), 26

interface software, 26

IOH, 37

IOL, 37

ionization radiation, 34

IP2, 37

IPI, 37

layers, 1

LEARN CONTROL, 4

LRN, 4, 16, 24

Madaline III learning algorithm, 20

Madaline Learning, 30

maximum dynamic range of weights, 18

MODE DESCRIPTIONS, 16

Mode Summary, 23, 24

MULTI-CHIP CONFIGURATIONS, 13

Multi-Chip Training, 26

multiplier, 5

Multiply Characteristics vs. VT Differential, 18

multiply-accumulate operations, 1

N0-N63, 3, 9

N0-N7, 16

N8-N15, 16

NeuroDynamX, 26

Neuron, 45

neuron body, 5

neuron cell, 5

Neuron Disable, 24

NEURON ENABLE, 3, 11, 13

Neuron Enable to Output Valid, 38

NEURON INPUTS, 3

NEURON OUTPUT, 4, 31

Neuron Output Transfer Characteristic Vs. SummingÊCurrentÊandÊVGAIN, 21

Neuron Output Transfer Characteristic Vs. SummingÊCurrentÊandÊVREF, 22

NEURON OUTPUTS, 3

Neuron Read (NR), 24

Neuron Read Mode, 21

neurons, 1

NjÐN63, 12

NMO, 4, 16, 21

NMO Signature, 24

non-linear resistive loads, 19

Non-Linear Resistive Summing Load, 19

NR, 21

NW vs NX, 24

OPERATING CONDITIONS, 37

operating modes, 23

Operating Temperature, 36, 37

ORDERING INFORMATION, 41

output, 5

Output Capacitance, 38

output current, 6

Output High Source Current, 37

Output Layer, 45

Output Leakage Current, 37

Output Low Sink Current, 37

Output Reference Voltage, 37

Output Short Circuit Current, 36

Output Slew Rate, 38

Output Valid (Feedback Input) to CLK High Setup, 38

outputs, 1, 5

Package, 40

Parallel Distributed Processing, 5, 8

Parallel Distributed Processing (PDP), 24

parallel processing, 1

parallel processors, 5

Pattern, 46

patterns, 8

Patterns Per Second, 1

PCPP, 26

PDP, 5, 8, 46

PDP/128 Inputs, 24

PDP/128-input mode, 9

PDP/64 Inputs, 24

PDP/64-input mode, 8

PDPÑParallel Distributed Processing Mode, 16

performance, 1, 33

Personal Computer PROM Programmer, 26

Perturb, 24

Pin Assignments, 41

PIN DESCRIPTIONS, 3

pin-to-pin interconnection, 13

Power Down, 24

PRINCIPLES OF OPERATION, 5

PROCESSING CONFIGURATIONS, 7

Processing Delay, 38

Prog Ref Column, 24

Prog Wt Column, 24

Program, 46

PROGRAMMING VOLTAGE SUPPLIES, 4

prototyping board, 13

QUER, 4, 24

QUERY, 4, 24

Radiation Exposure, 34, 35

Reduction of VT Shifts Through Bake-Train-Bake, 32

Ref Mod Decrease (RMD), 24

Ref Mod Increase (RMI), 24

Ref Vt Read (RVTR), 24

reference and gain control, 21

reference EEPROM cell, 17, 46

reference EEPROM cellÕ, 18

reference EEPROM cells, 15

Reference Modify and Read Modes, 18

REFERENCES, 42

Reset, 24

RESET FEEDBACK, 3

RESET INPUT, 3

RESETF, 3, 8, 9, 10, 24

RESETI, 3, 8, 9, 10, 24

RESETI/f High/Low to Valid Output, 38

resolution, 6

RMD, 18

RMI, 18

RVTR, 18

Sample and Hold Circuit Hold Time, 38

sample-and-hold features, 10

segmenting, 1

sigmoid, 22

Sigmoid Characteristic in VGAIN and HGAIN Modes, 22

sigmoid function, 1, 5, 46

sigmoid transfer characteristics, 21

sigmoid-gain control, 1

Signature Modes, 24

Signature Read, 16

simulation programs, 26

simulators, 30

single-layer feedforward network, 8

SMO+, 4, 19, 20, 21, 30

SMOÐ, 4, 19, 20, 21, 30

SMR, 19

SMR/P, 30

SSYN, 4, 24

Storage Temperature, 36

Sum Read (SMR), 24

Sum Read Mode, 19

Sum Read/Perturb, 30

Sum Read/Perturb (SMR/P), 24

sum-of-products differential current, 19

summing lines, 6

Summing Node Output Measurement, 20

Summing Nodes Selection and Output, 20

summing-node current input, 21

SUMMING-NODE OUTPUTS, 4

Synapse, 46

SYNAPSE CHARACTERISTICS, 15

Synapse Implementation, 7

Synapse Multiply Characteristics, 16

SYNAPSE OUTPUT, 4

Synapse Read (SYNR), 24

Synapse Read Mode, 21

SYNAPSE SELECT, 4

synapse weight, 17, 46

Synapse Weight Operation, 15

synapse weights, 26, 34

synapse-weight change, ÆVT, 27

synapses, 5

SYNO, 4, 17

SYNR, 21

TA, 37

TCE, 38

tCHCL, 38

tCL-H/CH-L, 38

technology, 6

tELQV, 38

Temperature and VCC Performance, 32

Temperature Sensitivity, 32

tHOLD, 38

threshold voltage, 15

threshold-voltage differentials, 15

tIH, 38

time-dependent input sequences, 11

tIVHH, 38

tIVP1H, 38

tIVQV, 38

TNE, 38

tNLQV, 38

TP, 38

tP1HP2H, 38

tP2HP2L, 38

tP2L-H/H-L, 38

tPLIV, 38

tQVCH, 38

TRAINING OPERATIONS, 26

Training System Interface Library, 26

Training Systems, 26

Training/Weight Setting Algorithm Equations, 29

transconductance values, 6

tRHQV, 38

TSIL, 26

TTL logic, 1

Tunneling, 15

Two-Layer Neural Network, 46

two-layer operation, 10

two-layer processing, 12

ui, 5

VCC, 21, 32, 33

VCC Active Supply Current, 37

VCC Sensitivity, 33

VCC Standby Supply Current, 37

VCC Supply Voltage, 36, 37

VfO, 38

VGAIN, 3, 31, 37

VIA, 37

VIH, 37

VIL, 37

vj, 5

VOA, 37

Voltage on Analog Inputs and, 36

Voltage on Digital Inputs, 36

VP1, 37

VP2, 37

VPL, 37

VPP1, 4, 17, 27

VPP1 High-Voltage Switch Voltage, 37

VPP1 Programming Supply Current, 37

VPP1 Setup Time Before VPP2, 38

VPP1, VPP2 Inactive/Low Voltage, 37

VPP1, VPP2 Recovery Time, 38

VPP1, VPP2 Voltages, 36

VPP2, 4, 17, 27

VPP2 Programming Supply Current, 37

VPP2 Pulse Rise/Fall Time, 38

VPP2 Pulse Width, 38

VPP2 Voltage Curves for Negative ÆVT Ñ Erase, 28

VPP2 Voltage Curves for Positive ÆVT Ñ Program, 27

VPP2 Weight Modify Pulse Voltage, 37

VPP2., 27

VREFi, 3, 21, 31, 37

VREFo, 3, 21, 31, 37

VT, 15, 17, 27, 29

VT Measurement System, 17

VTeo, 27, 29

VT_COM, 16, 18, 37

VT_DIFF, 15, 16, 31, 37

VT_real, 29

VT_REF, 16, 18, 37

VT_target, 27

VT_WT, 16, 18, 37

Weight, 37, 46

weight EEPROM cell, 17, 18, 46

weight EEPROM cells, 15

Weight Modify, 24

weight retention, 31

Weight Setting, 26, 46

Weight VT Read Mode, 17

WEIGHT-CELL SELECT, 4

Weight-Cycling Performance, 33

weight-modification voltage pulses, 15

Weight-Modify Decrease, 16

Weight-Modify Increase, 16

weight-setting algorithm, 27, 29, 30

weight-storage unit, 5

weights, 6

Wij, 5

WMD, 16

WMI, 16

WT, 4, 17, 26

Wt Mod Decrease (WMD), 24

Wt Mod Increase (WMI), 24

Wt Vt Read (WVTR), 24

WVTR, 17

ÆVFG, 7, 15

ÆVT, 27

Intel Neural Net Hot Line:  (408) 765-9235

Intel Neural Net Fax Line:  (408) 765-9797

FIGURES

*	 CHMOS is a patented process of Intel Corporation.